Ayesha Khalil, SP25-RCS-009
Research Proposal

3D Multi-UAV Navigation with Dynamic Target Assignment and Collision Avoidance

Background

Recent work on multi-UAV coordination using deep reinforcement learning has made progress on individual sub-problems: dynamic target assignment (DA-MAPPO, 2026), graph-based collision avoidance (IGAT-MARL, 2026), and 3D single-drone path planning. However, no existing study combines these three in one framework. DA-MAPPO is limited to three drones in a 2D environment. IGAT-MARL handles collision avoidance only, with no assignment component. Both papers explicitly list the missing combination as future work.

Research Problem

How can a swarm of five to ten UAVs navigate to dynamically assigned targets in a 3D environment while avoiding collisions with each other and with obstacles, using only local observations and no central controller at execution time?

Proposed Approach

The framework extends MAPPO with three additions. First, a minimum-cost Hungarian assignment runs at every decision step, and the assigned target's 3D position is fed directly into each drone's observation vector. Second, a conflict-aware interaction graph connects only drone pairs on a predicted collision course, keeping coordination sparse and relevant. Third, the action and observation spaces are extended to three dimensions with continuous velocity commands. Training follows a curriculum that begins with five drones and static targets, then adds moving targets, higher obstacle density, and larger swarm sizes up to ten drones.

Baselines for Comparison

Results will be compared against: standard MAPPO without assignment augmentation, DA-MAPPO ported to 3D without the collision graph, and IGAT-MARL ported with a fixed assignment. Each ablation directly tests the contribution of one component.

Timeline

| Period | Activity |
| --- | --- |
| Months 1-3 | Environment setup; baseline MAPPO in 3D validated |
| Months 4-7 | Core framework implementation; curriculum training |
| Months 8-11 | Full experiments across swarm sizes and obstacle densities |
| Months 12-15 | Writing and submission |

Tools and Resources

Python, PyTorch, PyBullet (simulation), MAPPO implementation. All tools are free and open source. No physical hardware required.

Key References

Sheng et al. (2026). Dynamic Target Assignment and Cooperative Decision-Making for UAV Swarms. IEEE IoT Journal.

Rezaee et al. (2026). Efficient Multi-Agent DRL for Multi UAV Collision Avoidance. Applied Soft Computing.

Fan et al. (2025). Dynamic Reward-Based DRL for UAV Path Planning in Large-Scale Environments.